1

TABLE OF CONTENTS

# Device safety instructions	4

- Disposal instructions	5

  - Packaging disposal	5

  - Water dispenser disposal	5

# General information on the water dispenser	5

- Water dispenser installation	20

  - Instructions for installation and connection to the power supply	20

  - Instructions for levelling the water dispenser	21

  - Water bottle installation instructions	23

  - Starting initialisation of the water dispenser when switching on the power	24

  - Instructions for moving the water dispenser	26

  - Instructions for loading cups	27

# Usage and management of the water dispenser	28

  - Tools of indication and management of the water dispenser	28

  - Water dispenser operation modes	29

  - Water dispenser operation mode selection	30

  - Water dispensing mode:	30

    - Hot water dispensing (accidental spillage protection on)	31

    - Hot water dispensing (accidental spillage protection off)	32

    - Cold water dispensing	32

    - Room temperature water dispensing	32

  - Energy saving mode (standby mode):	32

    - Smart energy saving mode	32

    - Energy saving mode activated when the console is connected	33

    - Connection of the energy saving console	36

    - Energy saving console	36

    - General recommendations on the use of the water dispenser energy saving mode	37

    - Programming energy saving console	37

  - Dispensed water temperature setting mode	46

# Description of the control functions of the water dispenser	50

  - Water bottle damage control	50

  - Indication of the hot water heater being turned on	50

  - Sensor to detect a user in front of the water dispenser	51

    - The full mode of user detection	51

    - The user detection only at a distance from water dispenser	51

    - User detection mode is switched off	53

  - Control of the water availability in the cold-water tank	53

  - Water level control in the droplet collector	54

  - Automatic control of the water dispenser being in work order	54

  - Water cooling control in the water dispenser	55

# Disinfection options for water dispenser during working	56

  - Sterilization of the cold-water tank	57

# Care and maintenance of water dispenser	57

  - Types of sanitization of water dispenser and their frequency	57

  - Cleaning of the external surfaces of the water dispenser	58

    - Cleaning the droplet collector	58

    - Cleaning the refrigerator condenser of the water dispenser	58

    - Cleaning of the internal surfaces of the water dispenser	58

  - Water drainage from the water dispenser	58

# The occurence of possible problems while using the water dispenser and their elimination	60

- Water dispenser labelling and packaging	62

  - Water dispenser labelling	62

  - Water dispenser packaging	64

# Shipping and storing	64

- Water dispenser service life	65

- Warranty of the manufacturer	65

  - Warranty terms	65

# Water dispenser delivery set	66

Appendix No 1. Symbols used in the text of the user manual	67

Appendix No 2. List of regulatory documents used in the user manuals	68

# Device safety instructions.

| WARNING!!! | To reduce the risk of fire, electric shock, or injuries arising from the use of your device, follow these basic steps: |
| --- | --- |
| Read all instructions before using the device. Never clean a water dispenser with flammable liquids; vapors may create a risk of fire or explosion. Do not store or use flammable liquids in the proximity of this or any other device; vapors may create a risk of fire or explosion. Do not connect the water dispenser to the power supply network until you are sure that the toggle switches of the water dispenser and of the water heating are in the off position. -SAVE THIS INSTRUCTION - |  |

General safety instructions:

- The water dispenser can be used by children aged 8 years and older and by persons with limited physical, sensory or mental abilities or with no experience or knowledge, if they are under the control of the person responsible for their safety, or have been instructed on the safe use of the water dispenser and understand the possible dangers. Children should not play with a water dispenser. Cleaning and maintenance of the device must not be carried out by children without supervision.

- Never open or disassemble the device yourself. Electric shock and burns may occur. If necessary and in case of malfunctions, contact an authorized service center.

- The water dispenser refrigeration unit contains a small amount of environmentally friendly refrigerant R134A. Ensure that during transportation, installation or movement, the tubes of the circuit through which the refrigerant circulates are not damaged. Spilled coolant splashes can cause eye inflammation or skin burns. In case if splashes got in the eyes, immediately contact a medical institution for medical help.

- The water dispenser must be connected to the power supply properly, according to these instructions (see the water dispenser installation section).

- With the water dispenser in place, make sure that the power cord is not pinched anywhere.

- The power outlet should be near the water dispenser, in an easily accessible place and not blocked by the device.

- Connect the water dispenser only to an individual outlet; it is strictly forbidden to use splitters or network extension cords.

- To disconnect the water dispenser from the power supply, use a power plug.

- Perform all manipulations with the water dispenser no earlier than 2 hours after disconnecting from the power supply, after the hot water and parts of the device have cooled. Danger of burns.

- When moving the water dispenser, be careful not to damage the refrigerant circuit and not to cause leakage of the refrigerant.

- Do not install a water dispenser near radiators or heat sources, as well as in places which are exposed to direct sunlight for a long time.

- In case of damage to the power cord, it must be replaced by the manufacturer, its service agent or other qualified person. It is prohibited to use the water dispenser with a damaged power cord.

- Before you start cleaning the device, turn off the water dispenser and remove the plug from the power outlet. Never pull the power cord of the device; only pull the plug.

- Never block the ventilation openings of the water dispenser.

- Never pour water into a droplets collector, in order to avoid burns with hot water splashes and to avoid spilling of water. The droplets collector is intended for collection of residual drops only.

- Do not pour hot water into containers not intended for hot water. Danger of burns.

- Do not tilt the water dispenser, which is full of water, for more than 10 degrees, in order to avoid water spillage and the unit’s failure.

- Before replacing the filters, the water dispenser must be disconnected from the power supply and hot water must be drained. Danger of burns.

- Sanitary treatment of the water dispenser must be carried out by an authorized service or by specially trained personnel. Danger of damage of the device and of burns.

- In case of a water dispenser malfunction, always contact an authorized service, which is the key to quality service and installation of original spare parts.

# Disposal instructions

## Packaging disposal

The packaging protects the water dispenser from damage during transportation. All materials used as packaging do not harm the environment and are recyclable. Please contribute to the protection of the nature by passing packaging  for environment-friendly recycling.

Warning:

    - Packaging material is not a plaything for children! While playing with packaging boxes and films, children can wrap themselves in them and suffocate.

## Water dispenser disposal

The decision to terminate the usage and to dispose the water dispenser is made by the owner of the water dispenser.

Disposal should be carried out in accordance with the legislative requirements of the country where the water dispenser is in operation.

This appliance has a sign in accordance with the guidelines of the directive WEEE 2012/19 / EU (WEEE – waste electrical and electronic equipment). This document contains instructions on the rules for the acceptance and disposal of the outdated equipment, operating within the EU.

An end-of-life device cannot be considered as useless garbage! Thanks to ecological recycling, it is source of valuable raw materials.

# General information on the water dispenser

Water dispenser is meant for:

- Cooling of drinking water.

- Heating of drinking water.

Forbidden:

- Use of any liquids other than drinking water in the water dispenser.

The water dispenser is made in climatic version UHL (for climatic category - boreal climate) category 4.2 according to GOST 15150-69 standard, and is designed for installation and operation in domestic premises at temperature of 10°C to 35°C, with a relative humidity of not more than 80% at 25°C, atmospheric pressure from 84 to 106,7 kPa.

Forbidden:

Usage and storage of a water dispenser at a temperature below freezing.

Note:

In case of freezing of water in the water dispenser, turn off the power supply of the device, move it to a heated room and contact an authorized service to diagnose and repair the water dispenser. In case of an emergency transportation or storage at temperatures below freezing, contact an authorized service to prepare a water dispenser for these operations.

According to the requirements of GOST 30804.3.2-2013 and GOST 30804.3.3-2013, the water dispenser operation does not interfere with the radiowaves.

The refrigerant circuit has been leak tested.

This water dispenser complies with the relevant safety requirements for electrical appliances in accordance with GOST IEC 60335-1-2015:

  - By type of protection against electric shock – class I,

  - By the case protection level – IP20 (IEC 60529)

  - By operating mode and operating conditions – long-term, without supervision.

We reserve the right to changes that do not affect the operation of the water dispenser.

- When upgrading a water dispenser, these specifications are subject to change without notice.

Features of dispensers specified in the manual:

Table 1

- «CT» - device name

- «G» - type of water supply to the water dispenser (cooler);

- «S» - version of the water dispenser (cooler) case;

- «X» - colour of the water dispenser (cooler) case;

- «Y» - type of water dispensed by water dispenser (cooler);

- «Z» - dimensions of the water dispenser (cooler);

- «F» - indicates the version or modification of the water dispenser (cooler);

- «M» - type of cooling of the water dispenser (cooler);

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| name | type of water supply | version of the case | colour of the case | type of water dispensed | dimensions | version or modification | type of cooling |
| CT | BOU (bottles water) | 02 (premium, aluminum case, round top glass, sensor USS) | SI (silver) | CHS (cold; hot; sparkling) | X (extra large; W365xD36 0xH1156) | changes that the operator must be informed of (for example, replacement of main units or components); V01,02,03….99 | 1C (single circuit device) |
|  |  |  | BL (black) | CHA (cold; hot; ambient) | L (large; W300xD37 0xH1055) |  | 2C (double circuit device) |
|  |  | 03 (business, metal case, even top glass, no USS sensor) | GL (gold) | CAS (cold; ambient; sparkling) | M (mini; W340xD36 0xH635) |  |  |
|  | POU (point of use) |  | BR (bronze) | A (ambient) | S (small; table top) |  |  |
|  |  |  | TI (titanium) | CA (cold; ambient) |  |  |  |
|  |  |  | CH (chocolate) | CHSA (cold; hot; sparkling; ambient) |  |  |  |

Table 2	Specifications of CTB-02XX-CHA -X - V0X - 1c

| Model | CTB-02XX-CHA -X - V0X-1c |
| --- | --- |
| Dimensions | 365 (W) x 360 (D) x 1156 (H) mm |
| Net weight | 37 kg |
| Compressor | Hermetic, high pressure 1/10 |
| Compressor overheat protection | 125 оC (automatic recovery) built-in |
| Refridgerator condenser | Pipeline, natural cooling |
| Refridgerator evaporator | Pipeline |
| Motor-compressor power | 74 W |
| Refrigerant | R-134A 40 g ± 2 g |
| Cold water tank | Steel AISI 316; 4 l |
| Single maximum dose of cold water | 2,0 l |
| Cooling control | Electromechanical thermostat |
| Cooling | Forced cooling |
| Water heater power | 500 W |
| Heated water tank | Steel AISI 316; 2.2 l |
| Amount of heated water | 12 glasses (80... 90) оC |
| Heater type | Tubular built-in heater |
| Heating control | Electronic thermostat |
| Heater overheat protection | Bimetal thermostat with manual recovery 115 оC |
| Cup holder capacity | 100 glasses 200 ml |
| Power supply voltage | 220-240 V~ 50 Hz |
| Power cord length | 2.5 m |
| The noise level generated by the device | Not more than -56 dB |
| Front panel | Glass |
| Case | Aluminium alloy EN AW6063-TG |
| Water tap | Noryl |

Table 3	Specifications of CTB - 03XX - CHA - X - V0X - 1c

| Model | CTB - 03XX - CHA - X - V0X - 1c |
| --- | --- |
| Dimensions | 340 (W) x 340 (D) x 1156 (H) mm |
| Net weight | 33 kg |
| Compressor | Hermetic, high pressure 1/10 |
| Compressor overheat protection | 125 оC (automatic recovery) built-in |
| Refridgerator condenser | Pipeline, natural cooling |
| Refridgerator evaporator | Pipeline |
| Motor-compressor power | 74 W |
| Refrigerant | R-134A 40 g ± 2 g |
| Cold water tank | Steel AISI 316; 4 l |
| Single maximum dose of cold water | 2,0 l |
| Water heater power | 500 W |
| Heated water tank | Steel AISI 316; 2.2 l |
| Amount of heated water | 12 glasses (80... 90) оC |
| Heater type | Tubular built-in heater |
| Heating control | Electronic thermostat |
| Heater overheat protection | Bimetal thermostat with manual recovery 115 оC |
| Nominal water pressure in the supply pipe | 0.3 – 0.5 MPa |
| Maximum water pressure in the supply pipe | 0.6 MPa |
| Regulation of water pressure in the device | Membrane type reducer 0.3 MPa |
| Cup holder capacity | 100 glasses 200 ml |
| Power supply voltage | 220-240 V~ 50 Hz |
| Power cord length | 2.5 m |
| The noise level generated by the device | Not more than -56 dB |
| Front panel | Glass |
| Case | Aluminium alloy EN AW6063-TG; carbon steel |
| Water tap | Noryl |

Table 4	Specifications of CTB - 02XX - CHA – L - V0X - 1c

| Model | CTB - 02XX - CHA – L - V0X - 1c |
| --- | --- |
| Dimensions | 300 (W) x 370 (D) x 1055 (H) mm |
| Net weight | 34 kg |
| Compressor | Hermetic, high pressure 1/10 |
| Compressor overheat protection | 125 оC (automatic recovery) built-in |
| Refridgerator condenser | Pipeline, natural cooling |
| Refridgerator evaporator | Pipeline |
| Motor-compressor power | 74 W |
| Refrigerant | R-134A 40 g ± 2 g |
| Cold water tank | Steel AISI 316; 4 l |
| Single maximum dose of cold water | 2,0 l |
| Cooling control | Electromechanical thermostat |
| Cooling | Forced cooling |
| Water heater power | 500 W |
| Heated water tank | Steel AISI 316; 2.2 l |
| Amount of heated water | 12 glasses (80... 90) оC |
| Heater type | Tubular built-in heater |
| Heating control | Electronic thermostat |
| Heater overheat protection | Bimetal thermostat with manual recovery 115 оC |
| Nominal water pressure in the supply pipe | 0.3 – 0.5 MPa |
| Maximum water pressure in the supply pipe | 0.6 MPa |
| Regulation of water pressure in the device | Membrane type reducer 0.3 MPa |
| Cup holder capacity | 100 glasses 200 ml |
| Power supply voltage | 220-240 V~ 50 Hz |
| Power cord length | 2.5 m |
| The noise level generated by the device | Not more than -56 dB |
| Front panel | Glass |
| Case | Aluminium alloy EN AW6063-TG; |
| Water tap | Noryl |

Table 5	Specifications of CTB – 03XX – CHA – M - V0X - 1c

| Model | CTB – 03XX – CHA – M - V0X - 1c |
| --- | --- |
| Dimensions | 340 (W) x 360 (D) x 635(H) mm |
| Net weight | 27 kg |
| Compressor | Hermetic, high pressure 1/10 |
| Compressor overheat protection | 125 оC (automatic recovery) built-in |
| Refridgerator condenser | Pipeline, natural cooling |
| Refridgerator evaporator | Pipeline |
| Motor-compressor power | 74 W |
| Refrigerant | R-134A 40 g ± 2 g |
| Cold water tank | Steel AISI 316; 4 l |
| Single maximum dose of cold water | 2,0 l |
| Water heater power | 500 W |
| Heated water tank | Steel AISI 316; 2.2 l |
| Amount of heated water | 12 glasses (80... 90) оC |
| Heater type | Tubular built-in heater |
| Heating control | Electronic thermostat |
| Heater overheat protection | Bimetal thermostat with manual recovery 115 оC |
| Nominal water pressure in the supply pipe | 0.3 – 0.5 MPa |
| Maximum water pressure in the supply pipe | 0.6 MPA |
| Regulation of water pressure in the device | Membrane type reducer 0.3 MPa |
| Cup holder capacity | 100 glasses 200 ml |
| Power supply voltage | 220-240 V~ 50 Hz |
| Power cord length | 2.5 m |
| The noise level generated by the device | Not more than -56 dB |
| Front panel | Glass |

Figure 1. Dimensions of the model CTB-02XX-CHA -X - V0X-1c

Figure 2. Dimensions of the model CTB - 03XX - CHA - X - V0X - 1c

Figure 3. Dimensions of the model CTB - 02XX - CHA – L - V0X - 1c

Figure 4. Dimensions of the model CTB – 03XX – CHA – M - V0X - 1c

Figure 5. Water dispenser model CTB-02XX-CHA -X - V0X-1c specification

Specification for Figure 5

| № | Name | № | Name |
| --- | --- | --- | --- |
| 1 | PRESS AGAIN indicator | 21 | Adjustable legs |
| 2 | OUT OF WATER indicator (no water supply to the system) | 22 | Cup holder |
| 3 | CALL SERVICE indicator (it is necessary to call service) | 23 | Upper glass panel |
| 4 | Hot water indicator | 24 | Bottle holder |
| 5 | Room temperature water indicator | 25 | Handle |
| 6 | Cold water indicator | 26 | DEVICE POWER SUPPLY switch |
| 7 | Hot water button | 27 | WATER HEATING switch |
| 8 | Room temperature water button | 28 | Removable power cord |
| 9 | Cold water button | 29 | Power plug with fuse |
| 11 | Control panel | 30 | Refrigerator condenser |
| 12 | Device case | 31 | Thermostat for water cooling control |
| 13 | Cup place | 34 | Water heater |
| 14 | Water tap | 35 | Hot water drain fitter |
| 15 | Droplets collector | 37 | Refrigerator compressor |
| 16 | Droplets collector grid | 38 | Back wall |
| 17 | Droplets collector overflow indicator | 43 | Ozone generator |
| 18 | Stopper for device movement | 44 | Filter for ozone distruction |
| 19 | Glass panel | 47 | External control panel |
| 20 | Rollers for device movement | 48 | External control panel socket |

Figure 6. Water dispenser model CTB-03XX-CHA -X - V0X-1c specification

Specification for Figure 6

| № | Name | № | Name |
| --- | --- | --- | --- |
| 1 | PRESS AGAIN indicator | 21 | Adjustable legs |
| 2 | OUT OF WATER indicator (no water supply to the system) | 22 | Cup holder |
| 3 | CALL SERVICE indicator (it is necessary to call service) | 23 | Upper glass panel |
| 4 | Hot water indicator | 24 | Bottle holder |
| 5 | Room temperature water indicator | 25 | Handle |
| 6 | Cold water indicator | 26 | DEVICE POWER SUPPLY switch |
| 7 | Hot water button | 27 | WATER HEATING switch |
| 8 | Room temperature water button | 28 | Removable power cord |
| 9 | Cold water button | 29 | Power plug with fuse |
| 11 | Control panel | 30 | Refrigerator condenser |
| 12 | Device case | 31 | Thermostat for water cooling control |
| 13 | Cup place | 34 | Water heater |
| 14 | Water tap | 35 | Hot water drain fitter |
| 15 | Droplets collector | 37 | Refrigerator compressor |
| 16 | Droplets collector grid | 38 | Back wall |
| 17 | Droplets collector overflow indicator | 43 | Ozone generator |
| 18 | Stopper for device movement | 44 | Filter for ozone distruction |
| 19 | Glass panel | 46 | External power saving control panel |
| 20 | Rollers for device movement | 47 | External power saving control panel socket |

Figure 7. Water dispenser model CTB-03XX-CHA -L - V0X-1c specification

Specification for Figure 7

| № | Name | № | Name |
| --- | --- | --- | --- |
| 1 | PRESS AGAIN indicator | 19 | Glass panel |
| 2 | OUT OF WATER indicator (no water supply to the system) | 20 | Rollers for device movement |
| 3 | CALL SERVICE indicator (it is necessary to call service) | 21 | Adjustable legs |
| 4 | Hot water indicator | 22 | Cup holder |
| 5 | Room temperature water indicator | 23 | Upper glass panel |
| 6 | Cold water indicator | 24 | Bottle holder |
| 7 | Hot water button | 25 | Handle |
| 8 | Room temperature water button | 26 | DEVICE POWER SUPPLY switch |
| 9 | Cold water button | 27 | WATER HEATING switch |
| 11 | Control panel | 28 | Removable power cord |
| 12 | Device case | 29 | Power plug with fuse |
| 13 | Cup place | 30 | Refrigerator condenser |
| 14 | Water tap | 31 | Thermostat for water cooling control |
| 15 | Droplets collector | 35 | Hot water drain fitter |
| 16 | Droplets collector grid | 38 | Back wall |
| 17 | Droplets collector overflow indicator | 47 | External control panel |
| 18 | Stopper for device movement | 48 | External control panel socket |

Figure 8. Water dispenser model CTB-03XX-CHA -M - V0X-1c specification

Specification for Figure 8

| № | Name | № | Name |
| --- | --- | --- | --- |
| 1 | PRESS AGAIN indicator | 18 | Stopper for device movement |
| 2 | OUT OF WATER indicator (no water supply to the system) | 19 | Glass panel |
| 3 | CALL SERVICE indicator (it is necessary to call service) | 21 | Adjustable legs |
| 4 | Hot water indicator | 22 | Cup holder |
| 5 | Room temperature water indicator | 23 | Upper glass panel |
| 6 | Cold water indicator | 24 | Bottle holder |
| 7 | Hot water button | 25 | Handle |
| 8 | Room temperature water button | 26 | DEVICE POWER SUPPLY switch |
| 9 | Cold water button | 27 | WATER HEATING switch |
| 11 | Control panel | 28 | Removable power cord |
| 12 | Device case | 29 | Power plug with fuse |
| 13 | Cup place | 30 | Refrigerator condenser |
| 14 | Water tap | 31 | Thermostat for water cooling control |
| 15 | Droplets collector | 35 | Hot water drain fitter |
| 16 | Droplets collector grid | 36 | Back wall |
| 17 | Droplets collector overflow indicator |  |  |

# Water dispenser installation

## Instructions for installation and connection to the power supply

The water dispenser is not supposed for outdoor installation.

    - Choosing premises

Install a water dispenser in a dry and ventilated area. The permissible ambient temperature for the correct operation of the water dispenser is from +10 °C to + 32 °C. The water dispenser should not be located near appliances that emit heat, for example, near heating radiators, stoves, etc., should not be exposed to direct sunlight.

    - Installing a water dispenser in places where water may get on the case is unacceptable.

Do not allow water on the water dispenser case, including when washing the water dispenser.

    - The water dispenser is connected to the power supply using the power cord. The plug socket (receptacle) must have a protective earthing, prescribed nominal (rated) voltage and frequency as indicated in the device specification. Electrical connections must be made according to the standards and regulations of the device location country. The water dispenser withstands short-term voltage drops, but no more than -10 to + 10% of the nominal voltage.

    - The use of sockets without protective earthing is unacceptable.

    - Before inserting the plug into the socket, make sure that the voltage and frequency indicated on the water dispenser data plate located on the back wall next to the movement handle match the frequency and voltage of your power supply. Deviations from -10 to +10% of the nominal voltage are permissable. Operation with a different power supply voltage and frequency is unacceptable, the water dispenser may be damaged.

    - The power outlet should be near the water dispenser, in an easily accessible place and not blocked by the device.

    - Connect the water dispenser only to an individual outlet; it is strictly forbidden to use splitters or power extenders.

    - To disconnect the water dispenser from the power supply, use a power plug.

    - It is not permissible to connect water dispenser to electronic, energy-efficient plugs or inverters that convert direct current into alternating current with a voltage of 230 V (for example, solar panels, marine electrical units, uninterruptible power supplies). The device may stop working.

    - The distance between the water dispenser and other heating elements must be not less than 300 mm, in case if that is not possible, it is necessary to use an insulating panel.

    - The distance between the back wall of the water dispenser and the wall must be at least 150 mm, so that the air outlet is not blocked.

    - Place the water dispenser on a firm, even, horizontal, and sound-absorbing surface.

    - Adjusting the vertical position of the water dispenser is made by two adjustable legs located in front of the unit.

    - Avoid installing the water dispenser in places where contact with water is possible.

    - Before connecting the water dispenser to the power supply or after moving, it is necessary to keep the device in a vertical position for at least 1 hour to avoid damage to the compressor of the device.

    - The water dispenser being switched on without hot and cold water tanks filled may cause damage to the device, always control the filling of tanks after switching the device on.

    - Turning the water heater on is allowed only after the hot tank is filled with water.

    - During operation of the water dispenser, the compressor and condenser located on the back wall heat up. Make sure that the water dispenser is installed exactly as recommended by this manual: insufficient ventilation may cause this device to work poorly or fail.

    - If the water dispenser was transported in a horizontal position, the oil from inside the compressor could flow into the refrigerant circuit. To allow the oil to drain into the compressor, set the device vertically and wait at least 2 hours before turning it on.

    - All manipulations inside the water dispenser case can be performed no earlier than 2 hours after the power supply is turned off, in order to avoid burns.

## Instructions for levelling the water dispenser

To level the water dispenser, in the front part there are adjustable legs, pos. No 21 Figure 5-8.

To adjust the height of the legs, tilt the water dispenser back, but for no more than 10 degrees, and adjust the legs.

Figure 9. Allowed tilt of the water dispenser during manipulations

Warning:

- Before adjusting the legs, disconnect the water dispenser from the power supply and water pipes, and wait at least 2 hours for the hot water and device parts to cool down. Danger of burns.

- Only adjust the height of the legs of the water dispenser by two persons, one person holding the device, the second person adjusting the height of the legs.

Note:

- Install the water dispenser on a horizontal and even surface, the adjustable legs are designed for minor adjustment. If it is impossible to adjust the horizontal position of the device with its legs, choose another place to install the dispenser.

- Unstable water dispenser may cause increased vibrations and noise while working.

- For installed water dispenser acceptable deviation from vertical is not more than 0.5 degrees.

Figure 10. Incorrectly installed water dispenser

Forbidden:

- Move, tilt, level the water dispenser connected to the power supply. Use additional gaskets and stoppers to level the water dispenser.

## Water bottle installation instructions

- Always use a suitable water bottle with a no-spill clack cap (No-Spill Cap). See Figure 11.

Figure 11. Cap with a clack.

- Do not use water bottles with any other type of cap. See Figure 12.

Figure 12. Examples of unsuitable caps.

3. Do not install water bottle without cap. See Figure 13.

Figure 13. Examples of water bottles installed without caps.

Forbidden:

  - To use water bottles with unsuitable types of caps

  - To use water bottles without caps

  - To use damaged water bottles

Correct installation of the bottle on the water dispenser:

- Take off the sticker from the bottle cap:

Figure 14. Protective sticker on the bottle cap.

Place the bottle on top of the water dispenser, lowering the bottle vertically downward on the opening finger.

Figure 15. Positioning the bottle when installing on the water dispenser.

The finger on which the bottle is placed automatically opens the clack (valve) in the cap, clack staying in the open position until the bottle is removed.

Figure 16. The location of the bottle cork clack on the finger of the water dispenser

When removing the bottle, the cork clack closes automatically, which contributes to its safe removal without spilling water, if it is inside.

Figure 17. The position of the bottle cork clack when the bottle is removed.

## Starting initialisation of the water dispenser when switching on the powe:

When the power of the water dispenser is switched on, the initial device initialisation is performed - automatic preparation for work. During starting initialisation the following is performed:

- Filling the water dispenser with water;

- Automatic water dispenser workability check;

- Primary heating and calibration of the hot water temperature sensor, for more precise adjustment of the hot water temperature and proper operation of the dispenser;

- Primary heating is performed to the water boiling point, therefore, in case of a program setting of the operating water heating temperature less than 94 ° C, a considerable time, up to 4 hours, is required to enter the operation mode;

- During starting initialization, it is possible to change the temperature settings of the dispensed water;

а. The first stage	b. The second stage

Figure 18. Starting initialization indication.

Starting initialization is split into 2 stages:

- The first stage, at this moment water supply to the user is impossible:

  - Filling the water dispenser with water if the water dispenser was empty. Checking the water dispenser fullness with water, checking the performance of the device;

  - Waiting to enter settings mode;

- The second stage, at this stage it is possible to use cold and room temperature water, hot water is unavailable:

  - Primary heating of hot water and testing the operability of the water heating system;

  - Calibration of the water heating sensor;

  - Quitting device initialization and beginning of operation.

Warning:

- Each time the power is turned on, the starting initialization of the water dispenser takes place;

- The starting initialization of the water dispenser takes a long time, about 1 hour, and if the device is switched on and off frequently, it can be inconvenient for users;

- Restrictions on the functionality of the water dispenser during starting initialization are not a malfunction;

Note:

- It takes about 2 hours to cool and heat the water after filling the tanks.

- The water dispenser enters normal operation after approximately 12 hours of operation.

Table 6. Types of alarms, depending on the location of the user

| 1. The user is at the water dispenser or the user presence sensor is disabled. |  |
| --- | --- |
| 2. User is at a distance from the water dispenser or only the near user presence zone is enabled for the sensor |  |
| 3. There is no user near water dispenser, or distance to the user is more than 1m |  |

## Instructions for moving the water dispenser

If it is necessary to move the water dispenser from one place to another, the device is equipped with rollers in the back part of the case.

To move, tilt the water dispenser back, but no more than 10 degrees. Move the device by holding the handle pos No 25 Figure 5-8, on the back part of the device.

If it is necessary to lift the water dispenser, use the handle pos. No 25 Figure 5-8 on the back part of the device, and the support for moving pos. No 18 Figure 5-8 on the frontal side. The design of water dispensers of the M series (see Figure 8) allowes only carrying the device or moving it on a transport cart.

Figure 19. Methods for moving a water dispenser.

Warning:

- If you need to move the water dispenser, unplug it from the power supply and wait for at least 2 hours to cool the hot water and parts of the device. Danger of burns.

- When moving over long distances, remove the bottle from the water dispenser and drain the water from the tanks to prevent water spilling and to reduce the weight of the device.

- Do not move the water dispenser on the rollers over long distances, you may damage the rollers. To move over long distances, use transport trolleys.

- Never use the condenser of the refrigerator unit and the internal parts of the device as a support to move the water dispenser, you can damage your hands and the device.

Forbidden:

- Move the water dispenser while plugged into the power supply.

## Instructions for loading cups

The water dispenser is equipped with a cup holder (pos. No 22 Figure 5-8) to hold no more than 100 cups with a diameter of 70 mm and a capacity of 200 ml.

Warning:

- Load the cups upside down after removing the packaging. Do not make much effort when loading cups.

- Use cups one by one, grabbing the bottom of the cup with your fingers.

- Do not use a cup holder to collect used cups.

- Do not use cups with a larger diameter; the water dispenser may be damaged.

- Do not use cups with a diameter smaller than 66 mm; cup holder will not hold them.

Figure 20.	 Cups loading.

# Usage and management of the water dispenser

## 5.1. Tools of indication and management of water dispenser

|  | Names |
| --- | --- |
| 1. | PRESS AGAIN indicator |
| 2. | OUT OF WATER indicator (no water supply to the system) |
| 3. | CALL SERVICE indicator (it is necessary to call service) |
| 4. | Hot water indicator |
| 5. | Room temperature water indicator |
| 6. | Cold water indicator |
| 7. | Hot water button |
| 8. | Room temperature water button |
| 9. | Cold water button |

Figure 21. Control panel indicators and buttons.

Figure 22. Water dispenser controls. Back view.

## Water dispenser operation modes

The device has 4 main modes of operation:

1) Water dispensing mode – the main mode of operation of the water dispenser. In this mode, water is dispensed to the user, the user presence in front of the device is monitored, if it is part of the functions of the selected model, all control functions of the device are in operation.

2) Energy saving mode (standby mode) – the dispenser operation mode set by the user, allowing user to turn off the water heating in accordance with user-defined settings. This mode is possible when connecting an external console in the water dispense mode.

3) Custom setting mode for dispensed water temperature. In this mode, you can change:

1. Hot water temperatures;

2. Room temperature water temperature;

3. Cold water temperature.

4) Service mode – this mode is available only to employees of the authorized service. Upon request, the authorized service can change the following operating parameters of the water dispenser:

1. Enable / disable protected hot water dispense mode;

2. Adjust the work of the sensor of user presence in front of the water dispenser, if it is included in the selected model;

3. Set the operation mode of the ozone sterilizer; Enable/disable intelligent power saving mode, if it is provided by the selected model.

Note:

- It is not recommended to switch operating modes without reading this operating manual, in order to avoid the disruption of the water dispenser.

- It is not recommended to change the water dispenser settings too often.

- It is recommended to have a responsible and trained person who monitors the status of the water dispenser and its settings.

## Water dispenser operation mode selection

The choice of operation mode is made after the water dispenser is turned on during the initial initialization.

    - The operating mode of water dispensing is set automatically, if no actions are taken during the initial initialization process.

    - Energy saving mode (standby mode) is set automatically when an external control panel with a preset on / off program is connected.

    - Intellectual power saving mode (if it is provided by the selected model) is set automatically, if it is enabled in the user settings.

    - To enter the custom setting of the temperature of the dispensed water, you must simultaneously press and hold all the water dispensing buttons during initial initialization.

Note:

- Starting state of the water dispenser before selecting the operation mode – the device is turned off.

- In case of accidental selection of an unnecessary mode of operation, turn off and then turn on the power supply of the water dispenser.

- Selection order of settings menu:

- the water dispenser power is turned on.

- the buttons for selecting the settings menu for temperature of the dispensed water are pressed, but no later than 8 seconds from the beginning of the initial device initialization.

If the settings menu has not been selected, turn off the power and repeat the selection of the settings menu again.

Warning:

- Before setting up the parameters of the water dispenser, it is necessary to carefully read this manual, to avoid malfunctions in the device.

- It is not recommended to change the water dispenser settings without agreement with all main users.

- The control of the technical condition of the water dispenser and the settings of the device should be carried out by the designated responsible person who has studied this manual.

## Water dispensing mode

The functions of the water dispenser in this mode:

1. Dispensing of water of choice to the user – hot water, cold water and room temperature water.

2. Ability to turn off the water heating.

3. Automatic water flow in the water dispenser.

4. The control functions of the water dispenser are performed automatically, without the participation of the user:

▪ Indication of hot water heater on;

▪ Automatic activation or deactivation of the energy saving mode, in case of the console installed;

▪ Determining the presence of a consumer in front of the water dispenser, if provided by the selected model;

▪ Monitoring the wholeness of the water bottle;

▪ Monitoring of water levels in a cold water tank and warning of emergency water levels;

▪ Monitoring and signaling the operability of water level sensors in a cold-water tank;

▪ Control of the turning on the ozone sterilizer for preventive disinfection of the cold-water reservoir;

▪ Water heating control and water heater performance check.

| № | Обозначения |
| --- | --- |
| 1. | PRESS AGAIN indicator |
| 2. | OUT OF WATER indicator (change the water bottle) |
| 3. | CALL SERVICE indicator (it is necessary to call service) |
| 4. | Hot water indicator |
| 5. | Room temperature water indicator |
| 6. | Cold water indicator |
| 7. | Hot water button |
| 8. | Room temperature water button |
| 9. | Cold water button |

Figure 23. Control panel indicators and buttons.

# Dispensing water to a user

Warning:

- Using containers not designed for hot water may cause burns and damage of the water dispenser.

- Do not turn off the accidental spillage protection function without absolute necessity – this will help protect the user from accidental burns.

Forbidden:

- Drain dirty water into a droplet collector;

- Operate a water dispenser with a full droplet collector;

- Use cup holder to collect used cups and debris;

## Hot water dispensing (accidental spillage protection on)

1. Approach the water dispenser from the front.

2. Put the water container under the water dispensing tap pos No 14 Figure 5-8

3. Press the hot water dispense button, pos. No. 7 – the indicator “PRESS AGAIN” pos. No. 1 should start to flash, then release the button. If you continue to hold the hot water dispense button for more than 8 seconds, the “PRESS AGAIN” indicator goes out and you need to repeat the action again to continue.

4. Press the hot water filling button again with the blinking “PRESS AGAIN” indicator and hold it until the required amount of water is dispensed, the hot water indicator pos. No. 4 lights up at maximum brightness and hot water is poured into the cup. After dispensing the right amount of hot water, release the water dispensing button. Pressing the other buttons deselects the hot water dispensing function.

Notes:

  - Accidental spillage control function protects from burns if you press the hot water dispensing button accidentally.

  - To turn on / off the accidental spillage protection function, contact an authorized service representative.

  - Pouring hot water into the container may occur with some time delay, but not more than 3 seconds, this is not a malfunction of the water dispenser.

# Hot water dispensing (accidental spillage protection off):

- Approach the water dispenser from the front.

- Put the water container under the water dispensing tap pos No 14 Figures 5-8.

- Press the hot water dispense button, pos. No. 7 and hold it until the required amount of water is obtained, the “hot water” indicator pos. No. 4 lights up at maximum brightness and hot water is poured into the cup.

- After dispensing the right amount of hot water, release the water dispensing button.

# Cold water dispensing:

- Approach the water dispenser from the front.

- Put the water container under the water dispensing tap pos No 14 Figures 5-8.

- Press the cold water dispense button, pos. No. 9 and hold it until the required amount of water is obtained, the “cold water” indicator pos. No. 6 lights up at maximum brightness and cold water is poured into the cup.

- After dispensing the right amount of cold water, release the water dispensing button.

# Room temperature water dispensing:

- Approach the water dispenser from the front.

- Put the water container under the water dispensing tap pos No 14 Figures 5-8.

- Press the room temperature water dispense button, pos. No. 8 and hold it until the required amount of water is obtained, the “room temperature water” indicator pos. No. 5 lights up at maximum brightness and cold water is poured into the cup.

- After dispensing the right amount of room temperature water, release the water dispensing button.

# Energy saving mode (standby mode):

    - Smart energy saving mode:

This water dispenser mode is not supported by every model.

Smart energy saving mode is a water dispenser operating mode that can be set according to the customer's request, allowing flexible control of water heating in accordance with the conditions of use of the water dispenser. The mode depends on the volume of hot water consumption and the presence of users in the device installation place. You must enable this mode in the user settings menu to activate it. In addition, in this mode the water circuit is disinfected periodically, which prevents biological contamination of the water dispenser.

This mode allows to get reduced energy consumption of the water dispenser in times of low and medium hot water consumption and when installed in spaces with a small number of people.

In case of reaching the maximum energy saving, when the water heating is completely turned off, an indication of the maximum energy saving appears, see figure 24. In this case, it may take some time to heat the water, about 1 hour after the first approach to the water dispenser, and water not hot enough at such time period is not a malfunction.

# Warning:

- The time required to heat the water when exiting the smart energy saving mode is one hour. This should be taken into account when operating a water dispenser with the smart energy saving mode enabled.

- Not all models of water dispensers support this mode.

- To enable smart mode, you must contact the authorized service.

- After turning on the smart energy saving mode, its operation is fully automatic and cannot be adjusted by the user.

# Energy saving mode, activated when the console is connected.

Energy saving mode, or the so-called “sleep” mode, is a water dispenser mode set as per customer request, allowing to turn off the water heating functions in accordance with user-defined settings. To activate it, an external control panel should be connected in the water supply mode. In addition, in this mode the water circuit is disinfected periodically, which prevents biological contamination of the water dispenser.

Figure 24. Displaying turned on energy saving mode

The functions of the water dispenser in this mode:

Turning off the water heating, which significantly reduces energy consumption of the water dispenser:

- Switching water heating on and off occurs automatically according to a user-defined program, which lets turning the water dispenser on and off when necessary.

- Automatic sanitization of the water dispenser (periodic sterilization of the air cavity of a cold-water tank using ozone-air mixture) and automatic activation of heating once a day, which helps prevent biological contamination of water in water tanks.

- Energy saving mode is turned off (without disconnecting the console from the water dispenser) after the water dispenser turns off the heating and cooling of water.

# Warning

      - The time required to heat the water when exiting the energy saving mode is one hour. This should be taken into account when programming the console and temporarily turning off the energy saving mode.

Temporary switching off (without reprogramming of the console) can be done when short-term use of water dispenser takes place, for example, on a weekend. It is not necessary to program the console for this, it is enough to press any button of the water dispense on the water dispenser once.

Figure 25. Temporary switching off of the energy saving mode.

After that, the energy saving mode will be suspended and in 1.5 hours (90 minutes) after the last water dispensation, the water dispenser will return to the energy saving mode.

To permanently disable the energy saving mode (see Figure 26) (without reprogramming the console and disconnecting the console from the water dispenser), use the MODE button on the console and select ON.

To re-enable the energy saving mode, use the MODE button on the

control panel to select AUTO ON or AUTO OFF mode (see Figure 27).

Figure 27. Automatic activation of energy saving mode.

Permanent switching on of the energy saving mode is carried out under the condition of connecting the console, without programming the on and off time - using the MODE button, the OFF mode is selected on the console (see Figure 28).

Figure 28. Energy saving mode is permanently on.

To exit this mode, please follow the rules from p. 1 or 2.

# Note:

      - The energy saving mode can work only when an external console is connected;

      - External energy saving console is not part of a mandatory delivery when purchasing a water dispenser; to purchase it please contact an authorized service.

# Forbidden:

      - Connection of any devices to the input socket of an external control panel, with the exception of a genuine external control panel.

# Connection of the energy saving console

Figure 29. Connection of the energy saving console.

# Energy saving console:

The energy saving console purpose is to turn on the energy saving mode and set the on / off time for heating and cooling of water.

After the console is connected to the water dispenser, the indicator of the console connection to the device will light, indicating that the console is ready to work.

| № | Name |
| --- | --- |
| 1 | SET - selection of the parameter to be set. |
| 2 | SYS1 - service mode button |
| 3 | SYS2 - service mode button |
| 4 | MODE – mode selection ON, AUTO ON, OFF, AUTO OFF |
| 5 | TIME – switch to time mode |
| 6 | < – change time, days of the week, select SYS2 mode for time programming. |
| 7 | > – change time, days of the week, choice of program number for time programming. |
| 8 | RESET - resetting the console to zero values |
| 9 | Indicator of connection of the console to the water dispenser. |
| 10 | Display |
| 11 | Plugging connector of the console |
| 12 | Plugging wire of the console |

Figure 30. Energy saving console.

# General recommendations on the use of water dispenser energy saving mode:

- Define the responsible person who will program or connect the energy saving console.

- Use as less of remote control timers as possible: this will let to avoid errors when programming the console and will let to use the energy saving mode as efficiently as possible.

- Before programming the energy saving console, make the most convenient schedule for all users of the water dispenser, to avoid future conflict situations.

- Set the energy saving mode to switch off one hour before the possible use of the water dispenser. This time is necessary for heating and cooling water before use.

- It is recommended to switch off the power saving mode after possible use of the water dispenser, for example, at the end of the working day.

Applying these recommendations will ensure that you are comfortly using water dispenser with the highest possible energy efficiency.

# Programming energy saving console:

Programming of the energy saving console can be performed both if it is connected to the water dispenser or when it is not connected - this will not affect the performance of the console.

Figure 31. View of the energy saving console.

Buttons of energy saving console.

- SET — selection of the parameter to be set.

- SYS1 — service mode button

- SYS2 — service mode button

- MODE — mode selection ON, AUTO ON, OFF, AUTO OFF

- TIME – switch to time mode

- < – change time, days of the week, select SYS2 mode for time programming.

- > – change time, days of the week, choice of program number for time programming.

- RESET — resetting the console to zero values.

# Set the current time.

- In the time display mode, press and hold the SET button for at least 3 seconds – the days of the week indicator will appear.

# Figure 32.

- Using the button "<" or ">" select the current day of the week: MO, TU, WE,  TH, FR, SA, SU.

# Figure 33.

- Press the SET button to save the current day of the week and to select the current time - the clock indicator will start to blink.

# Figure 34.

- Using the button "<" or ">" set the current hour 1, 2, 3, 4, ….. 23, 24.

# Figure 35.

- Press the SET button to save the current hour, and then proceed to setting minutes – the minutes indicator will start blinking.

Figure 36.

- Using the button "<" or ">" set the current minutes 00, 01, 02, 03, … , 58, 59.

# Figure 37.

Press the SET button to save the selected values and switch to time mode.

# Figure 38.

Set winter / summer time

In time mode, press and hold the TIME button. Using the “<” button, set the summer time to +1 hour or remove summer time.

Figure 39.

# Selecting the time display mode 12 HOURS (12-hour) or 24 HOURS (24-hour)

To change and switch to a new mode, press and hold the TIME button for 3 seconds, and then release it.

12-hour time mode selected 			24-hour time mode selected

Figure 40.

# Mode selection ON, AUTO ON, OFF, AUTO OFF

Energy saving console modes.

ON – the water dispenser is constantly on in normal operation mode with the function of heating and cooling of water; console software settings do not work.

AUTO ON – the water dispenser is controlled using the console software settings. The priority setting is heating and cooling of water. So, for example, if in the program No. 1, according to the schedule, the device must be in the energy saving mode, and in the program No. 2 – in the working mode, then in this case the priority will be to the working mode of the water dispenser.

OFF – the water dispenser is constantly on in energy saving mode, and the function of heating and cooling of water is turned off; console software settings do not work.

AUTO OFF – the water dispenser is controlled using the console software settings. The priority setting is the energy saving mode, in which the function of heating and cooling of water is disabled. So, for example, if in program No. 1, according to the schedule, the device must be in energy saving mode, and in program No. 2 – in the working mode, then with this setting the priority will be to the energy saving mode with the functon of heating and cooling of water turned off.

The choice of the mode is made by pressing the MODE button in the time mode on the console.

|  | Mode ON – energy saving mode off |
| --- | --- |
|  | Mode AUTO ON – energy saving mode on in auto mode (see AUTO ON mode description) |
|  | Mode OFF – energy saving mode on |
|  | Mode AUTO OFF – energy saving mode off in auto mode (see AUTO OFF mode description) |

# Figure 41.

Reset all console settings

. Using a thin object, press the RESET button, after which all console settings will be reset.

# Figure 42.

Console Timer Programming

The energy saving console allows to program 20 independent timers to control the operation of the water dispenser.

# What can be set up:

- Day of the week or a combination of days of the week when the energy saving mode is on or off.

Possible combinations of days of the week: MO, TU, WE, TH, FR, SA, SU

  - Specific day of the week – for example, MO

  - Weekdays from Monday to Friday - MO, TU, WE, TH, FR

  - Saturday, Sunday - SA, SU

  - Weekdays from Monday to Saturday - MO, TU, WE, TH, FR, SA.

  - Monday, Wednesday, Friday - MO, WE, FR

  - Tuesday, Thursday, Saturday — TU, TH, SA

  - Monday, Tuesday, Wednesday - MO, TU, WE

  - Thursday, Friday, Saturday - TH, FR, SA

  - All week - MO, TU, WE, TH, FR, SA, SU

- Time of turning energy saving mode on and off.

- Timers programming routine

- Programmed timer number

Use the ">" button to select the timer number to program. The indicator of the selected timer will blink.

# Figure 43.

To start programming, select the 1ON timer using the “>” button and press the SET button to program the days of the week, while the days of the week indicator blinks.

# Figure 44.

Using the button < or > select the required day of the week or a combination of the days of the week.

# Figure 45.

Press the SET button to save the selected day of the week and go to the setting the operation beginning hour for the device – the hour indicator will start blinking.

# Figure 46.

Using the < or > button, set the hour for switching on the device, using hour range from 0 to 24 hours.

# Figure 47.

Press the SET button to save the hour and go to the setting the operation start minute for the device – the minutes indicator will start blinking.

# Figure 48.

Using the < or > button, set the start minute within the range from 00 to 59 minutes.

Figure 49.

Press the SET button to save the set timer parameters – the timer number will blink on the screen, and you can reprogram this timer or switch to the next timer programming using the < or > button.

Figure 50.

# By pressing the TIME button, you can return to the time mode.

Quick reset of timer parameters (timer deactivation).

Use the ">" button to select the number of the timer that you want to reset.

# Figure 51.

Press the MODE button: the timer will be reset. At the same time, the previous value of the timer will be saved in the memory of the console, and it can be returned by pressing the MODE button again.

# Figure 52.

Service mode SYS1

# This mode of operation of the energy saving console is designed to control the performance of the water dispenser. This mode of operation can only be used by authorized service personnel.

# Warning:

  - This mode of operation of the energy saving console can be used only by an authorized service. To prevent incorrect operation of the water dispenser, do not use this function yourself

  - If you accidentally select this mode, set a different mode of operation of the power saving console.

When pressing SYS1 button, service mode SYS1 is entered.

Figure 53.

To exit this mode, press the SYS1 button again. If you find that the service mode SYS1 is on, to exit it, also press the SYS1 button.

Figure 54.

# Service mode SYS2

# This mode of operation of the energy saving console is designed to control the work of the     water dispenser. This mode of operation can only be used by authorized service personnel.

# Warning:

  - This mode of operation of the energy saving console can be used only by an authorized service. To prevent incorrect operation of the water dispenser, do not use this function yourself

  - If you accidentally select this mode, set a different mode of operation of the power saving console.

- Indication of the service mode SYS2

If you accidentally enter this mode and the corresponding indicators appear on the power saving console screen (as shown below), press the TIME button to exit this mode, after which you will return to the clock mode. Check the required operation settings of the console.

Random access to this mode is possible by pressing the < button.

# Figure 55.

- Indication of the service mode SYS2 in programming mode (possible options). To exit this mode, you should:

# Figure 56.

Press the button SYS2

# Figure 57.

Press the TIME button, after wich you will return into the clock mode. Check console settings which are essential for operation.

Figure 58.

# 5.6. Dispensed water temperature setting mode

This menu is selected after the water dispenser is turned on and the initial initialization indication appeared, by simultaneously pressing and holding all the water dispensing buttons until the menu entry appears, but no later than 8 seconds after the beginning of indication. If the menu entry did not occur, it is necessary to turn off and turn the dispenser on again.

To select a submenu, one of the water dispensing buttons should be pressed. Pressing the same water dispensing button again sets up the functions of this submenu, pressing another water dispensing button opens the next submenu. When entering the submenu on the console, the preset value indicator is lit.

a. Mode selection	b. Mode indication

Figure 59. Dispensed water temperature setting mode selection.

In this mode of operation the following can be done:

- Dispensed hot water temperature selection;

- Dispensed cold water temperature selection;

- Dispensed room temperature water temperature selection.

To select the hot water temperature, press the hot water dispense button, the hot water indicator will start blinking and the previously set temperature value will be displayed, then by next pressing of the hot water dispense button, the following temperature values can be selected, the accuracy of the temperature setting is ± 2° С:

Figure 60. Setting of the hot water temperature.

- 76оС – indicator “CALL SERVICE” pos. No 3 is on.

- 84оС – indicator “OUT OF WATER” pos. No 2 is on.

- 94оС – indicator “PRESS AGAIN” pos. No 1 is on.

- Water heating is turned off, regardless of the position of the “Water heating” switch pos. No. 27 – indicators are off.

Recommendations for choosing the hot water temperature:

- 94оС – temperature is preferred for making coffee, black tea, bouillon cubes, noodles and instant porridge;

- 84оС – temperature is preferred for making green tea;

- 76оС – temperature is preferred for making white tea;

Water heating is turned off – if it is necessary to completely eliminate the presence of hot water for the safety of using the water dispenser, without possibility to turn on the heating with an external switch.

To select the cold-water temperature, press the cold water dispense button, the cold-water indicator will start blinking and the previously set temperature will be displayed, then pressing the cold water dispense button, you can select the following temperature values, the needed water temperature is obtained by mixing cold and hot water, depends on several parameters and can only be an approximate guideline:

Figure 61. Setting of the cold-water temperature.

- 7оС – indicator “CALL SERVICE” pos No 3 is on.

- 15оС – indicator “OUT OF WATER” pos No 2 is on.

- 20оС – indicator “PRESS AGAIN” pos. No 1 is on.

- Water mixing is turned off, the water is as cold as possible - the indicators are off.

To select a room temperature water, press the room temperature water dispense button, the room temperature water indicator will start blinking and the previously set temperature will be displayed, by pressing the water dispense button at room temperature, you can select the following temperatures, the desired water temperature is obtained by mixing hot and cold water, depends on several parameters and can only be an approximate guideline:

Figure 62. Setting of the room temperature water temperature.

- 20оС – indicator “CALL SERVICE” pos No 3 is on.

- 25оС – indicator “OUT OF WATER” pos No 2 is on.

- 37оС – indicator “PRESS AGAIN” pos No 1 is on.

# Note:

- When dispensing cold and room temperature water in mixing mode, there may be characteristic clicks or changes in water flow when dispensing water;

- When dispensing cold and room temperature water in mixing mode, the temperature parameters of the water can correspond to the selected values only when at least 100 grams of water are dispensed.

To exit this menu, after performing the necessary manipulations or in case of an erroneous entry:

- Turn off and then turn on the power of the water dispenser;

- If there were no actions or the display of the main menu is on, the water dispenser will switch to the main water dispensing mode after 40 seconds.

# Warning:

- Before setting up the parameters of the water dispenser, it is necessary to carefully study this user manual, which will help to avoid failures in the operation of the device;

- It is not recommended to change the water dispenser settings without agreement with all main users;

- The control of the technical condition of the water dispenser and settings of the device must be carried out by the designated responsible person who has studied this user manual.

# Description of the control functions of the water dispenser.

  - Water bottle damage control:

The water dispenser is equipped with a water bottle damage automatic control function made to detect damaged bottles. Operation of the device with damaged bottles is forbidden and may lead to device failure.

Figure 63. Indication of a damaged bottle (overflow of the cold-water tank).

# Warning:

    - If an indication of a damaged bottle appears, replace the damaged water bottle with a new one as soon as possible, to prevent water from spilling out of the water dispenser;

    - A damaged bottle may not have any visible signs of damage;

    - After removing the damaged bottle from the device before installing the new bottle, dispense at least 1.5 liters of cold water to stop the indication of the damaged bottle to appear;

    - In case of reappearance of the indication with a new bottle – remove the bottle from the device, turn off the water dispenser and contact the authorized service.

# Forbidden:

    - Usage of the water dispenser with a working indication of a damaged bottle;

    - Re-installation of the water bottle with which indication of a damaged bottle appeared on the dispenser.

# Indication of the hot water heater being turned on:

The water dispenser is equipped with an indication function for the hot water heater being turned on.

If the heater is turned off, the water is not heated, and the hot water supply to the user is blocked. At the same time the indicator "Hot water" is switched off.

After turning on the hot water heater using the “Water heating” switch, pos. No 27 Figure 5-8, water is heated and water is dispensed to the user.

In the event of water heating being turned off, to prevent biological contamination of the hot water tank, water is automatically heated for disinfection every 24 hours.

Heater off	Heater on

Figure 64. Heater state indicator

# Sensor to detect a user in front of the water dispenser:

In the water dispenser, the function of determining a user at different distances is implemented:

- The presence of the user in the immediate proximity of the water dispenser – the determining area from 0 to 0.5 m from the device.

- The presence of the user at a distance from the water dispenser – the determining area is from 0.5 to 1 m from the device.

- The absence of the user in front of the dispenser – user is further than 1 meter from the device.

The user detection sensor in front of the water dispenser can be configured for 3 modes of operation:

- The full mode of user detection.

- The user detection only in the immediate vicinity of water dispenser.

- User detection mode is switched off.

# The full mode of user detection.

This mode of the user detection sensor is the most optimal for the operation of the water dispenser, with all possible functions of the device being included.

- The user is outside the control zone – the water dispenser is in standby mode.

# Figure 65.

- The user is in the distant zone of determination, 0.5 - 1 m from the water dispenser – water dispenser goes into standby mode and activates some of its functions

# Figure 66.

- The user is in the near-field zone of determination, 0 - 0.5 m from the water dispenser – the water dispenser goes into active mode and activates all its functions.

# Figure 67.

    - The user detection only at a distance from water dispenser.

It is recommended to switch to this mode of the user detection sensor in places with a lot of people walking past the water dispenser at a distance of 0.5 to 1 m.

The user is outside the control area (0.5 m and further) – the water dispenser is in standby mode.

# Figure 68.

- The user is in the close proximity area, 0 - 0.5 m from the water dispenser – the device goes into the active mode and uses all its functions.

# Figure 69.

    - User detection mode is switched off.

It is recommended to switch to this mode when the water dispenser is installed in corridors up to 1.5 meters wide and there there are a lot of people passing by the device.

1. Indication of this mode of the water dispenser, constantly on.

# Figure 70.

  - Control of the water availability in the cold water tank:

The water dispenser has an automatic function to control the water availability in the cold-water tank. The water level in the dispenser may fall below the required, the device stops dispensing water and signals the absence of water. The operation of the water dispenser is restored automatically when a new bottle of water is installed and the device is filled with water. Indication of the water being below the required level in the water dispenser.

# Figure 71.

Warning:

    - If the indication of the water absence appears after turning on the water dispenser with the water bottle installed, turn off the power supply of the water dispenser and contact an authorized service representative.

    - If the indication of the water absence appears and it is impossible to change the water bottle, leave an empty bottle in the water dispenser to prevent a leakage of the ozone-air mixture.

Indication of the water absence may be caused by the following situations:

    - Bottle is out of water – replace the bottle with a new one.

    - If there is water in the bottle – water dispenser malfunction. Disconnect the device from the power supply, remove the bottle and contact the authorized service.

# Water level control in the droplet collector:

To control the level of water in the droplet collector there is a pop-up float. When the float ascends, it is necessary to drain the water from the collector otherwise water can get inside the water dispenser and damage it.

Figure 72. The position of the float when the collector is full.

# Automatic control of the water dispenser being in work order:

There are several build-in functions of automatic control of water dispenser health. In case of water dispenser malfunctions, the “CALL SERVICE” indicator, pos. No 3 lights up and all functions are blocked.

# Figure 73.

If the water dispenser signals on a malfunction, it is necessary to re-check the device. It is necessary to turn off and turn on the water dispenser with switch "Power supply" pos. No 26 Figure 5-8. When re-indicating a fault, disconnect the water dispenser from the power and water supplies and contact an authorized service.

# Warning:

    - Do not attempt to repair water dispenser by yourslef; it is life threatening.

    - Limit the possibility for others to turn on a faulty water dispenser. For example, stick the tag “Faulty” or use other available methods.

# Water cooling control in the water dispenser:

The water-cooling control in the water dispenser is made by an electromechanical thermostat.

Figure 74. Location of thermostat of water-cooling control in the water dispenser.

Figure 75. Sticker on thermostat of water-cooling control.

Turning the thermostat pointer towards "+" increases the average temperature of the water in the cold water tank, turning the thermostat pointer towards "-" accordingly reduces the average water temperature.

To adjust the temperature of the thermostat, use only a special key

.

Figure 76. Thermostat adjustment key.

Using other tools may damage the water dispenser. The thermostat location helps to limit access to its adjustments for unprepared persons and children.

# Warning:

    - It is not recommended to change the factory setting of the position of the water-cooling control thermostats, since this setting is optimal for operation in normal conditions and ensures maximum energy efficiency of the water dispenser.

    - It is not recommended to change the thermostat settings frequently, as this may damage the water dispenser.

# Forbidden:

    - To adjust thermostat settings by persons who have not been trained accordingly, in order to avoid the water dispenser failing.

# 7. Disinfection options for water dispenser during working:

The water dispenser has two options for disinfecting the device:

- Ozone sterilizer for disinfecting the cold-water tank and the air cavity of the water bottle.

- Periodically forced heating of water in the hot water tank, in the case of water heating being off.

# Sterilization of the cold water tank:

In order to prevent a decrease in water quality, the water dispenser is equipped with an ozone sterilization device for a cold-water tank.

The sterilization device operation modes can be changed in the service mode, only by an authorized service worker.

# Table 7. Recommendations on the choice of operating modes of the ozone sterilizer

| Mode No | Mode Name | Recommendations on the usage of the chosen mode |
| --- | --- | --- |
| 1 | Off | In case of intolerance to ozone. |
| 2 | Turns on when a user presence sensor triggers. | The most complete treatment, it is recommended to use in public places and at high risks of biological contamination of the water dispenser, including the rare usage of the device. |
| 3 | Turns on when you press the water dispense buttons. | It is recommended to use in public places with a very high flow of people and intensive use of the water dispenser. |
| 4 | Cyclic operation with time intervals. | When using water dispenser with user presence sensor switched off and in case of high sensitivity to ozone. |

Warning:

    - During operation of the water dispenser, a faint odor of ozone may be present.

    - If a persistent and pronounced odor of ozone appears during the operation of the water dispenser, turn off the ozone sterilizer and contact an authorized service center.

# Care and maintenance of water dispenser:

Warning:

    - Cleaning of the water dispenser surfaces should be done only after disconnecting from the power supply network

# Types of sanitization of water dispenser and their frequency:

Table 8. Water dispenser sanitization frequency

| Sanitary treatment of water dispenser with tanks disinfection. | At least once in 6 months or if the water dispenser has not been used for 3 weeks. |
| --- | --- |
| Cleaning of the internal surfaces | At least once in 6 months or as needed. |
| Cleaning the bottle holding funnel* | Before each installation of the bottle, using light disinfectants. |
| Cleaning the droplet collector* | Daily, using light disinfectants. |
| Cleaning the tap * | Daily, using light disinfectants. |
| Cleaning of the external surfaces* | As becoming unclean. |
| Cleaning of condenser grid* | At least once a month or as needed. |

    - * This treatment can be done by the user, other types of treatment can only be performed by an authorized service.

# Cleaning of the external surfaces of the water dispenser:

Cleaning the external surfaces of the water dispenser is done manually using hand-washing liquid added to warm water. After cleaning the external surfaces, it is necessary to reliably remove the remnants of detergents, and wipe them dry after, using a soft tissues or cloth to avoid damaging the surface of the water dispenser.

# Forbidden:

To clean the water dispenser case with water jets.

# Warning:

    - Cleaners containing abrasives, acids, alkalis, and solvents can damage external surfaces of the water dispenser.

Water used should not:

    - Get into cup holders pos. No 22 Figure 5-8.

    - Get on the back side of the water dispenser.

    - Get inside the water dispenser while droplet collector is taken off pos No 15 Figure 5-8.

# Cleaning the droplet collector:

Carry out regular cleaning of the droplet collector, pos. No 15 Figure 5-8, using light disinfectants, after having taking the collector out of the water dispenser case. Cleaning must be done manually, as washing in a dishwasher can damage the surface of the collector.

Watch the filling of the droplet collector with water and pour water out of it as needed, when water is spilling out of it into the water dispenser, its failure is possible.

# Cleaning the refrigirator condenser of the water dispenser:

The accumulation of dust on the refrigerator condenser will affect the operation of the water dispenser. It is necessary to clean from dust the device refrigerator condenser, pos. No. 30 Figure 5-8. Clean the condenser of the refrigeration unit from dust, possibly using a soft non-metallic brush or vacuum cleaner. Using wet wipes and hard items to clean the condenser can damage the water dispenser.

8.2.3. Cleaning of the internal surfaces of the water  dispenser:

Warning:

      - It is recommended to start cleaning the internal surface of the water dispenser only after at least 2 hours after disconnecting from the power supply network, after cooling of the internal parts of the water dispenser.

      - Cleaning of the internal surfaces of the water dispenser can be done only by a specialist from the authorized service or by a specially trained person.

# 8.3. Water drainage from the water dispenser:

Water drainage from the water dispenser is done in two stages:

- Water drainage from a cold-water tank is made with the water dispenser power supply on.

- Water drainage from a hot water tank is made with the water dispenser power supply off and cooling time of at least 2 hours.

Figure 77. Water drainage from a hot water tank.

# Forbidden:

- Drain water from a water dispenser by persons who don’t have necessary training.

Note:

  - The first step is to always drain the water from the cold-water tank, then from the hot-water tank.

# 9.The occurrence of possible problems while using the water dispenser and their elimination:

If you have any questions while using the water dispenser, contact the authorized service for advice, this will help you to avoid many usage problems.

| № | Issue | Reason | Solution |
| --- | --- | --- | --- |
| 1 | The control panel indicators are off. | Water dispenser power off | Make sure that there are no warning signs about the reason of the power off and turn on the water dispenser. |
|  |  | Absence of power in the electricity supply network | When issue is solved, turn on the water dispenser. |
|  |  | Malfunction of the water dispenser power cord. | Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
|  |  | Malfunction of the water dispenser. | Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 2 | Hot water is cold | Water heater off. | Turn on the water heater and wait until the water heats up, if after 1 hour the water remains cold, it is a malfunction. Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
|  |  | Water heater on. | Wait for 1 hour, perhaps the heater was just turned on. If after 1 hour the water remains cold, it is a malfunction. Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 3 | Hot water is not hot enough, less than 70 °С. | High consumption of hot water. | Water does not have time to heat up. Wait for the water to heat up. Wait for 1 hour, perhaps the heater was just turned on. If after 1 hour the water remains cold, it is a malfunction. Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 4 | Hot water dispensing delay for less than 5 seconds | With the rare usage of hot water, a small air chamber is formed in the hot water tank. | This is not a malfunction and cannot damage the water dispenser. |
| 5 | Hot water dispensing delay for more than 15 seconds | Hot water tank is not filled with water. | Water dispenser malfunction – disconnect from the power supply, contact the authorized service. |
| 6 | Cold water is not cold, compressor is working. | High consumption of cold water. | Wait for 1 hour, perhaps the heater was just turned on. If after 1 hour the water does not cool down, it is a malfunction. Disconnect the water dispenser from the power supply and water |

|  | Cold water temperature is higher than 12° С. |  | supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| --- | --- | --- | --- |
| 7 | Cold water is not cold, compressor is not working. Cold water temperature is higher than 12° С. | Water dispenser malfunction. | Check the cold-water temperature settings. If dispensed water matches the settings, the water dispenser is working, otherwise disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 8 | Water dispenser work is very noisy. | Water dispenser is installed incorrectly. | Install the water dispenser vertically on a stable base. |
|  |  | Water dispenser is installed on the vibrating base. | Install the water dispenser on the sound absorbing coating. |
| 9 | Touching the water dispenser causes static electricity to discharge. | A large accumulation of static electricity on the human body. | This is not a malfunction. |
| 10 | Frequent false indication of a malfunction. | Water dispenser is not stable, and can move during operation. | Install the water dispenser vertically on a stable base. |
|  |  | Moving the water dispenser without disconnecting from power. | When moving, disconnect the water dispenser from the power supply. |
| 11 | There is no dispensing of any water when buttons are pressed, the indicators for dispensing water light up | Water dispenser malfunction. | Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 12 | When buttons are pressed, the water indicators do not light up. | Simultaneously pressing 2 and more buttons. | Position the finger on only one button at a time. |
|  |  | Inaccurate finger positioning on the button. | Position the finger on only one button at a time. |
|  |  | Wet surface of the control panel. | Wipe the control panel glass. |
|  |  | Wet surface of the control panel. | Condensate may form on the control panel when hot water is added. Wipe the control panel glass. |
|  |  | Water dispenser malfunction. | Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |
| 13 | Spontaneous water dispensation. | Wet surface of the control panel. | Wipe the glass surface of the control panel. |
|  |  | Water dispenser malfunction. | Disconnect the water dispenser from the power supply and water supply, contact an authorized service for troubleshooting, restrict access to the faulty device. |

- Water dispenser labelling and packaging:

# Water dispenser labelling:

Location of the main identification label on the water dispenser.

Identification label

Hot water heater switching

Water dispenser switching

Regulatory Disposal Information

Burn hazard warning

Ozon generator label

Identification label on the package

Shipping Information

Figure 78. Label examples

# Water dispenser packaging:

The water dispenser is packaged at the factory and includes:

    - Plastic bag – 1 pc.

    - Shipping packaging:

      - Cardboard box – 1 pc.

      - Cardboard pallet – 1 pc.

      - Packing tape – 2 pc.

Operational documentation is packaged in a plastic bag and placed inside the water dispenser.

# Shipping and storage:

- Water dispensers in shipping packaging should be transported by closed road, rail or sea transport in accordance with the rules applicable to that type of transport.

- Placement and fastening of water dispensers in vehicles should ensure their stable position, exclude the possibility of the displacement of water dispensers and their hits against each other, against the walls of shipping packaging and vehicles.

- Water dispensers should be stored on racks in the manufacturer's packaging in an upright position, and stacking of water dispensers is forbidden.

- The distance from the water dispenser in a packaging, placed on the rack, to any object (including the walls and the floor of the storage) should be at least 10 cm.

- The distance from the water dispenser in a packaging to the heater must be at least 50 cm.

- Water dispensers storage conditions:

  - air temperature should be from 5° С to 40° С;

  - relative air humidity should be no more than 80% at 25° С.

- The room air must be free from dust, acid and alkali vapors, as well as from corrosive gases.

# Water dispenser service life:

Water dispenser service life is 10 years.

# Attention:

- After the service life of the water dispenser has expired, the manufacturer is not responsible for the safe operation of the water dispenser.

Further operation may be unsafe, since the probability of electric and fire hazardous situations due to the natural aging of materials and the wear of the water dispenser components increases significantly.

# Warranty of the manufacturer:

The warranty period for the water dispenser is 12 months from the date of sale, if the client observes the usage rules set in this user manual, as well as the requirements for shipping and storage of water dispensers.

The elimination of defects discovered during the warranty period must be carried out by manufacturer free of charge.

# Warranty terms:

    - Certificate:

The warranty is valid only with a valid warranty certificate. Warranty certificate is invalid if it contains errors, inconsistencies and corrections.

The following positions must be filled in the Warranty certificate:

Name of the product;

Model;

Serial number of the product;

Date of sale;

Seller’s signature;

Stamp of trade organization.

In the absence of the date of sale, the warranty period is automatically calculated from the date of manufacturing of the water dispenser.

    - Shipping and storing:

      - Any defects of the water dispenser caused by careless transportation and storage of the water dispenser are not subject of free warranty repair.

    - Installation and operation:

Free warranty repair is not performed if a water dispenser defect is caused by:

      - Incorrect connection of the water dispenser to the power supply and water supply networks, as well as inconsistency of the parameters of the above networks with the parameters set by mandatory state standards and operating instructions.

      - The use of a water dispenser not in accordance with the operating instructions, as well as careless usage, resulting in the occurrence of mechanical or other types of defects.

      - Use of a water dispenser not in accordance with its intended purpose or in inappropriate operating conditions.

13.1.4. Repair:

Free warranty repair is not performed if a water dispenser:

      - Has traces of outside interference or repair by unauthorized persons.

      - Has unauthorized design changes or components installed in the dispenser that differ from those used by the manufacturer.

      - Has damages caused by unpredictable phenomena and forces of nature.

      - Has damages caused by using non-standard and unacceptable consumables and accessories.

      - Has damage caused by the foreign objects, aggressive substances, animals and insects gotten into the water dispenser.

      - Does not have a serial number or or it can not be defined.

      - Warranty does not cover consumables and accessories.

    - Responsibility:

      - The manufacturer is not responsible for any damage caused to you and others due to violation of the rules of the operating manual, especially caused by improper connection to the power supply and water supply, as well as inconsistency of the parameters of the above networks with the parameters imposed by mandatory state standards and by operating manual, unauthorized repairs, water dispenser usage not for its intended purpose.

# 14.Water dispenser delivery set:

- Water dispenser – 1 pc.

- User manual – 1 pc.

- Package – 1 pc.

Appendix No 1. Symbols used in the text of the user manual.

Appendix No 2. The list of regulatory documents used in the user manual.

WEEE 2012/19/EU The Waste Electrical and Electronic Equipment Directive

GOST 15150-69 – GOST 15150-69 – Machines, devices and other technical products. Modifications for different climatic regions. Categories, operating, storage and transportation as to environment climatic aspects influence (with Amendments No 1, 2, 3, 4, 5).

GOST 30804.3.2-2013 – (IEC 61000-3-2:2009) Electromagnetic compatibility of technical equipment. Harmonic current emissions (equipment input current ≤ 16 A per phase). Standards and test methods.

GOST 30804.3.3-2013 – Еlectromagnetic compatibility of technical equipment. Limitation of voltage changes, voltage fluctuations and flicker in public low-voltage supply systems. Equipment with rated current ≤ 16 A per phase and not subject to conditional connection. Limits and test methods.

IEC 60529:2013 Degrees of protection provided by enclosures (IP Code)

EC 89/336/EEC Electromagnetic Compatibility (EMC)

EN 60335-2-75 Household and similar electrical appliances